| 1 | Model | TY-FC240D-01C （24 inch self checkout kiosk） |  |
| --- | --- | --- | --- |
| 2 | General specification | Mainboard | SV1a-H8112P-B |
|  |  | RAM | 4G/8G/16G（optional） DDR3-1600/1333MHz |
|  |  | Storage memory | 64G/128G（optional）MSATASSD;256GSSD |
|  |  | Operating system | Window 7/window 10 |
|  |  | Network support | 1000M Ethernet、2.4GWiFi/bluetooth、4G optional |
| 3 | panel specification | Panel specs | 24inch Touch Pro |
|  |  | Panel type | TFT-LCD |
|  |  | Backlight type | WLED |
|  |  | Panel size | 527.04 x 296.46 mm |
|  |  | Resolution | 1920*1080 |
|  |  | Contrast(typ) | 1000:1 |
|  |  | Visual angle | 89/89/89/89（typ） |
|  |  | Brightness (typ)/(min) | 250nits（No touch），225nits（touch） |
|  |  | Touch Technology | Capacitive |
|  |  | Touch points | 10 points |
|  |  | working voltage | DC 12v |
|  |  | Operating temperature | 0°C ~ 40°C, 20% ~ 80% RH. |
|  |  | Storage temperature | -20°C~ 60°C, 10% ~ 95% RH |
|  |  | Interface | VGA , HDMI， |
| 4 | motherboard specs | Mainboard | SV1a-H8112P-B |
|  |  | RAM | 4G/8G/16G（optional） DDR3-1600/1333MHz |
|  |  | Storage memory | 64G/128G（optional）MSATASSD;256GSSD |
|  |  | Operating system | Window 7/window 10 |
|  |  | Network support | 1000M Ethernet、2.4GWiFi/bluetooth、4G optional |
|  |  | Processor | Intel@4 generationsI3/I5/I7 CPU, LGA1150 |
|  |  | Chipset | Intel® H81 Chipset, TDP 4.1W |
|  |  | BIOS | AMI UEFI BIOS |
|  |  | Interface | 2*USB2.0，2*USB3.0，HDMI，DC-IN,VGA,LAN，PS/2 |
|  |  | Serial port | 2*RS232. |
|  |  | Built-in USB port | 4*header |
|  |  | Ethernet | Adaptive 1000MEthernet |
|  |  | LVDS Output | Support Dual Channel 24bit, max resolution up to 1920*1200@60Hz |
|  |  | VGA Output | Support max resolution up to 1920*1200@60Hz |
|  |  | HDMI Output | Support max resolution up to 4096*2304@24Hz |
|  |  | Audio output | Realtek ALC662 5.1 Channel HDA Codec , Support MIC/Line-out Ports 1 * Amplifier, 6-W (per channel) into an 8-Ω Loads. |

| 5 | Printer | Printing method | Thermal printing |
| --- | --- | --- | --- |
|  |  | Resolution | 203 dpi (8 dots/mm) |
|  |  | Paper width | 80（mm） |
|  |  | Printing width | 72（mm） |
|  |  | Print Density | Level 1-4 ,adjustable |
|  |  | interface | RS-232/USB |
|  |  | printing speed | 170（mm/s）max |
|  |  | Detection function | temperature detection and PNE sensor |
|  |  | working voltage | 24V DC |
|  |  | working temperature | ’-10℃~+50℃/10% RH ~ 80% RH |
|  |  | Cutter life | 700000 times |
|  |  | TPH life | 50km |
| 6 | Scanner | working voltage | 5v DC |
|  |  | working temperature | -20°C to 60°C |
|  |  | working current | 300mA |
|  |  | Support code system | Australian Post, Aztec, BPO, Canada Post, Codabar, Codablock, Code 11, Code 39, Code 93, Code 128/EAN128, DataMatrix, Dutch Post, EAN.UCC Composite, Interleaved 2 of 5, Japan Post, Matrix 2 of 5, MaxiCode, MicroPDF417, MSI Code, PDF417, Planet, Plessey Code, Postnet, QR code, RSS, Standard 2 of 5, Telepen, TLC 39, UPC/EAN. |
|  |  | Reading mode | barcode / QR code |
|  |  | Reading accuracy | ≥8mil |
|  |  | lighting | Visible red light |
|  |  | luminous intensity | 630nm + 10nm |
|  |  | Scanning angle | 47°（Horizontal), 38° (vertical) |
|  |  | Scanning speed | 3m/s |
|  |  | Resolution | 640*480 pixel |
|  |  | feedback | Buzzer |

| 7 | 3D Face recognition camera | Power supply mode | USB2.0 |
| --- | --- | --- | --- |
|  |  | baseline | 75mm |
|  |  | Recognition distance | 0.8-6m |
|  |  | Depth range | 0.35-1m/0.6-8m（meter） |
|  |  | Resolution | depth map 1280*1024@7FPS/1280*720@30FPS color map |
|  |  |  | 640*480@30FPS/320*240@30FPS Depth picture color picture |
|  |  |  | 160*120@30FPS Depth picture |
|  |  | Power consumption | 2.5wMAX ≦500mA |
|  |  | Accuracy | 1m：±1-3mm |
|  |  | DepthFOV | H 58.4º V 45.5º |
|  |  | Data transfer | USB2.0 or above |
|  |  | Video interface | UVC mode (no need driver) |
|  |  | Color FOV | H 66.1º V 40.2º |
|  |  | Delay | 30ms-45ms |
| 8 | Environment Specs | work environment | Indoor |
|  |  | working temperature | 0°C - 40°C |
|  |  | Operating humidity | 10%-90%(No condensation) |
|  |  | Storage temperature | ‘-20°C - 50°C |
|  |  | Storage humidity | 10%-90%(No condensation) |
| 9 | accessories | Paper | 1 |
|  |  | Key | 2 |
|  |  | Power cable | 1 |
| 10 | Electrical parameter | input voltage | Rated voltage： 90（V/ac）~264（V/ac），50/60（Hz） |
|  |  | Maximum power | 50W |
| 11 | Mechanical characteristics | Appearance | Supermarket cashier + PC |
|  |  | Cabinet material | metal+ stainless steel |
|  |  | outline dimension | As shown picture |

Dimension：(the plastic bags bracket is not included)