| 1 | Model | TY-FC270D-01A |  |
| --- | --- | --- | --- |
| 2 | General specification | motherboard | SV1a-H8112P-B |
|  |  | RAM | 4G |
|  |  | Storage memory | 64GB SSD |
|  |  | OS | Windows 10 |
|  |  | Network support | 1 * Realtek GbE LAN Chip (10/100/1000 Mbps) |
| 3 | panel specification | Panel specs | 27inch |
|  |  | Panel type | TFT-LCD |
|  |  | Backlight type | WLED |
|  |  | Panel size | 597.888*336.312mm（W*H） |
|  |  | Resolution | 1920*1080 |
|  |  | Contrast(typ) | 1000:1 |
|  |  | Visual angle | H:178°, V:178° |
|  |  | Brightness (typ)/(min) | 300nits |
|  |  | Touch Technology | Capacitive |
|  |  | Touch points | 10 points |
|  |  | working voltage | DC 5v |
|  |  | Operating temperature | 0°C ~ 50°C, 20% ~ 80% RH |
|  |  | Storage temperature | -20°C~ 60°C, 10% ~ 95% RH |
|  |  | Interface | VGA , HDMI，USB, LAN |
| 4 | motherboard specs | processor | Intel Pentium/ Celeron/ 4th Generation Core i5 CPU, LGA1150 |
|  |  | Chipset | Intel® H81，TDP 4.1 W |
|  |  | Memory type | Support DDR3 1600/1333 MHz, 2 * SO-DIMM Slot, Up to 16GB |
|  |  | interface | 8 * USB: 2 * Rear I/O(USB3.0/USB2.0) + 2 * Rear I/O(USB2.0) + 4 * Header(USB2.0) 8 * GPIO 1 * PS/2 2 * Mini PCI-E [4] 2 * DC19V In Connector [5] |
|  |  | BIOS | AMI UEFI BIOS |
|  |  | Audio | Realtek ALC662 5.1 Channel HDA Codec , Support MIC/Line-out Ports 1 * Amplifier, 6-W(per channel) into an 8-Ω Loads |

| 5 | Printer | Printing method | Thermal printing |
| --- | --- | --- | --- |
|  |  | Resolution | 576 dpi (8 dots/mm) |
|  |  | Paper width | 80（mm） |
|  |  | Printing width | 72（mm） |
|  |  | interface | USB |
|  |  | printing speed | 150（mm/s）max |
|  |  | Detection function | temperature detection and PNE sensor |
|  |  | working voltage | 24V DC |
|  |  | working temperature | ’-10℃~+50℃/10% RH ~ 80% RH |
|  |  | TPH life | 50km |
| 6 | Scanner | working voltage | 5v DC |
|  |  | working temperature ` | -20°C to 60°C |
|  |  | working current | 75mA-100mA |
|  |  | Support code system | 1D SUPPORT: industrial 25、 standard、 matrix 25、 china post 25、code 11、codabar、 MSI/Plessey、 code39(standard 39 & full ASCII code 39)、code 32、code 93、code128、UPCA、UPCE、EAN13、EAN8、UPC/EAN、 add-on 2/5、ISBN、ISSN、GS1(RSS) RSS-14、LIMITED、 EXPENDED、include GS1 STACK 2D SUPPORT: pdf417, data matrix ,qr code . ect. |
|  |  | Reading mode | CMOS |
|  |  | Reading accuracy | ≥8mil |
|  |  | lighting | LED (6000K) 90lux @100mm |
|  |  | luminous intensity | 265 LUX (130 mm) |
|  |  | Scanning angle | Omnidirection scan |
|  |  | Scanning speed | 30 times / s |
|  |  | Resolution | 640*480 pixel |
|  |  | feedback | Buzzer |

| 7 | Environment Specs | work environment | Indoor |
| --- | --- | --- | --- |
|  |  | working temperature | 0°C - 40°C |
|  |  | Operating humidity | 10%-90%(No condensation) |
|  |  | Storage temperature | ‘-20°C - 50°C |
|  |  | Storage humidity | 10%-90%(No condensation) |
| 8 | accessories | setscrew | 4 |
|  |  | keys | 2 |
|  |  | certificate of qualification | 1 |
|  |  | Warranty card | 1 |
|  | accessories | setscrew | 4 |
| 9 | Electrical parameter | input voltage | Rated voltage： 90（V/ac）~264（V/ac），50/60（Hz） |
|  |  | Maximum power | ≦155 |
| 10 | Mechanical characteristics | appearance | Front white spray, black spray, with black stand |
|  |  | Cabinet material | Mild Steel front shell + galvanized mounting plate + Mild Steel rear shell |
|  |  | outline dimension | 462*191*1812.9(mm) |
|  |  | Packing size | Kiosk(wooden package）：120*30*68cm Pole(carton package）：87*31*33cm |
|  |  | Product weight | GW：kiosk 65KGS Pole 15KGS |
| 11 | Host port description |  |  |
|  |  | .DC19V Power Input Connector 1 .VGA Connector .HDMI Connector [1] .USB 2.0 Connectors USB 3.0 Connectors LAN Connector Audio Connectors |  |

Photo：

Drawing：